# Abstract

Section owners: Prashant, Ankit, Sharvesh. Convention: draft last. Status (2026-09-30): rewritten on the executed modules, including the a09 fleet interval. Every value cites [CL-xx]. Kashmir-Division scope; the legacy "342 permits → 207 routes / ~1,009 fleet / Srinagar-metropolitan" framing is superseded and barred. ~230 words.

Most Indian cities that need bus-network reform lack the origin–destination and ridership data that route-optimisation methods assume. We present an open-data, demand-free framework for bus route rationalisation and fleet sizing and apply it to Kashmir Division, India (6,584,762 residents) [CL-11], using a gridded population surface, OpenStreetMap, a routing engine and the digitised permit register. Its 614 route records (445 private permits and 169 timetabled services) describe only 157 corridors, so the plan's apparent 71 % "route reduction" is 99.4 % corridor retention plus a change of unit [CL-01, CL-02, CL-07, CL-08]. Measuring catchments on the pedestrian network rather than as straight-line buffers lowers population served by a median of 37.4 % per route and division coverage from 35.5 % to 24.2 % [CL-26, CL-28]; only 10.4 % of residents live near a 15-minute service [CL-42]. Against 2,526 driver-GPS service runs, modelled run times are too fast and a per-kilometre cap, not demand, sets cycle time on 169 of 186 routes [CL-18, CL-31]. Joint uncertainty analysis therefore reports the fleet as a range: 989–1,058 buses as specified, but 1,130–1,266 at observed pace, against a published 1,011; the service hierarchy is robust (97.8 % tier agreement) [CL-56, CL-57]. Thirty per cent of the fleet, bought in order of new residents reached per bus, delivers 92 % of the plan's coverage [CL-61]. The plan is decision-robust in its hierarchy, not demand-validated.

Keywords: bus route rationalisation; demand-data scarcity; open data; network walk catchments; fleet sizing; uncertainty analysis; Kashmir