# 8. Conclusions

Section owners: Ankit, Prashant, Sharvesh. Target length: ~400 words. Status: complete draft.

This study rationalised an inherited stage-carriage network entirely from open data and with no demand feed. Spatial analysis of the register showed it to be a permit register, not a route register: 614 route records (445 permits and 169 timetabled services) [CL-01] describe 157 distinct corridors [CL-02], which the plan reorganises into 186 active routes (32 trunk, 154 feeder) sized at a 1,011-vehicle fleet (187/754/70 HPV/MPV/LPV; +68.5 % over the operating baseline) [CL-06, CL-36]. The pipeline is reproducible end to end under a fixed seed.

The central methodological contribution is to measure catchments on a walkable pedestrian network rather than as straight-line buffers. This single change lowers measured population served by a median of 37.4 % per route and cuts headline division coverage from 35.5 % to 24.2 % [CL-26, CL-28] — because network catchments are the exception, not the rule, in the rationalisation literature, the correction is roughly a third of the benefit that Euclidean methods report.

The study's other contributions are contributions of honesty. It shows the apparent 71 % "route reduction" is 99.4 % corridor retention plus a change of unit [CL-07, CL-08]; it recovers an undocumented tourist multiplier that had injected 285,914 synthetic residents into a coverage numerator [CL-30]; and it demonstrates that an intended sanity cap silently governs cycle time on 90.9 % of routes while sitting below real pace [CL-31, CL-32]. Carried through a joint uncertainty analysis, that finding turns the published 1,011 buses into the floor of a range: 1,130–1,266 once urban and peri-urban run times follow observed pace [CL-56]. The service hierarchy, by contrast, survives (97.8 % tier agreement) [CL-57], and a third of the fleet buys nine-tenths of the coverage [CL-61].

On validation we are deliberately narrow. Supply-side GPS reproduces the geometry and urban moving speed [CL-33, CL-35] and the fleet arithmetic reproduces exactly [CL-55], but the operator benchmark fails its pre-registered band and is not independent [CL-54], driver GPS carries no ridership signal, and the demand-side channels — an expert weight elicitation and an on-street boarding survey — remain near-term work (§6.4). The plan is therefore decision-robust, not demand-validated — the strongest honest claim available to a demand-free method, and one we intend to strengthen with the field corroboration set out above.

Because the four inputs — a gridded population surface, OpenStreetMap, a routing engine, and a permit list — exist for most Indian cities without an organised bus network, the framework transfers directly, with data maturity treated as a ladder whose rungs §7.5 ranks by the uncertainty each removes.

The sentence the whole design works to keep unwriteable is that the composite index bears no established relationship to travel demand. Four directions would close the gap: (i) a stratified boarding-count survey to test the index-demand link directly; (ii) the deferred Delphi/AHP elicitation to ground the weights; (iii) wider GPS instrumentation to replace the binding cap with measured pace; and (iv) explicit seasonal and dynamic demand modelling for the tourist economy the resident raster cannot see.